Kinetic Study of Curcumin on Modal Fabric.

## Abu Naser Md. Ahsanul Haque     Manwar Hussain     Shamima akter Smriti     Fahmida Siddiqa     Nawshin Farzana

## Abstract

A kinetic study of curcumin on modal fabric was carried out using an initial dye concentration of 1 g/L at three different temperatures, 70 °C, 85 °C and 100 °C, at pH 7 and an material to liquor ratio of 1:20. Pseudo first-order and pseudo second-order kinetics were applied, and it was found that the adsorption kinetics of curcumin on modal fabric matched the pseudo second-order kinetic model. The activation energy was found to be equal to 71.14 kJ/mol, while the enthalpy and entropy of activation were 68.16 kJ/mol and -66.02 J/molK respectively.