Evaluation of Radioactivity in Soil and Rock Samples from an Undiscovered Sea Beach in the Southeastern Coastline of Bangladesh and Associated Health Risk

Author:  M M Mahfuz Siraz, Md Hossain Kamal, Zulfiqar Hasan Khan, M S Alam, Jubair Al Mahmud, Md Bazlar Rashid , Mayeen Uddin Khandaker , Hamid Osman, S Yeasmin

Abstract:

This study marks the first-ever assessment of radiological hazards linked to the sands and rocks of Patuartek Sea Beach, situated along one of the world's longest sea beaches in Cox' Bazar of Bangladesh. Through the utilization of an HPGe detector, a comprehensive analysis of the activity concentrations of 226Ra, 232Th, and 40 K was conducted, and their activity ranged from 7 to 23 Bq/kg, 9-58 Bq/kg, and 172-340 Bq/kg, respectively, in soils, and 19-24 Bq/kg, 27-39 Bq/kg, and 340-410 Bq/kg, respectively, in rocks. Some sand samples exhibited elevated levels of 232Th, while the rock samples displayed higher levels of 40 K compared to the global average. The radiological hazard parameters were assessed, and no values surpassed the recommended limits set by several international organizations. Hence, the sands and rocks of Patuartek sea beach pose no significant radiological risk to the residents or tourists. The findings of this study provide crucial insights for the development of a radiological baseline map in the country, which is important due to the commissioning of the country's first nuclear power plant Rooppur Nuclear Power Plant. The data may also stimulate interest in the rare-earth minerals present in the area, which is important for the electronics industry, thorium-based nuclear fuel cycles.

Conference / Journal Link:

https://pubmed.ncbi.nlm.nih.gov/37558890/

DOI:

10.1007/s10661-023-11636-5